THERMODYNAMICS

Estimation of Diffusion Coefficients in Bitumen Solvent Mixtures Using X-ray CAT Scanning and Low Field NMR

Wen, Y.1, Kantzas, A.1,2, Wang, G. 1,2
1Department of Chemical and Petroleum Engineering, University of Calgary, Calgary, Alberta, Canada, 2Tomographic Imaging and Porous Media Laboratory, University of Calgary, Calgary, Alberta, Canada
Diffusion plays an important role in both VAPEX and Solvent Injection processes for oil/bitumen recovery. In this paper, X-ray CAT scanning and low field NMR have been used to obtain experimental data for the diffusion of several solvents in bulk bitumen.

Low field NMR has been used successfully in determining both solvent content and viscosity reduction in heavy oil and bitumen mixtures with various solvents.  As a solvent comes into contact with a heavy oil or bitumen sample, then the mobility of hydrogen bearing molecules of both solvent and oil changes.  These changes are detectable through changes in the NMR relaxation characteristics of both solvent and oil.  These changes can be correlated to mass flux and concentration changes.

X-ray CAT scanning takes advantage of density contrasts in the scanned sample through the measured CT number. With solvent diffusing into heavy oil or bitumen, the CT number changes during the process and provides the corresponded density of the mixture. Therefore, the concentration gradient distribution with distance can be obtained. A Fick type of equation can be written and an apparent diffusion coefficient can be calculated.  This approach and the results of this analysis are presented along with an evaluation of the applicability of the assumptions in Fick’s Law.

During this paper, diffusion coefficient in the solvent-bitumen system was considered both as a constant and as a function of concentration. Two models have been built and were compared against experimental results from CAT scanning and NMR. It was found that consisted diffusion coefficients could be obtained from these data.

