# Geomentric Dimensioning and Tolerancing  (GD&T)

## Introduction

- So far, tolerancing has referred to allowed variations in the values of dimensions on the drawing.

- Geometric Dimensioning and Tolerancing (GD&T) is a system of drawing mark-up symbols used to reduce ambiguity and uncertainty about the desired geometric shape

- GD&T system consists of geometric symbols, reference entities, and tolerance frames

- NOT every edge or surface on a part has to be treated – only the ones critical to design intent

| - Consider this part with a basic dimension and tolerances - What are the possible variations in the size of this dimension? |  |  |  |  |
| --- | --- | --- | --- | --- |
| The minimum | The maximum |  | A tapered edge? |  |
| Effects of manufacuting #1? | Effects of manufacuting #2? |  | Effects of manufacuting #3? |  |

- Is it permissible for the person producing the part to decide which of the previous shapes is acceptable?

  - Must not waste time creating geometry that is more accurate than necessary.

  - However, creating critical geometry insufficiently accurately will result in rejects.

  - Either case costs $$$!

- So, how are\can these types of features specified?

- To specify geometric tolerancing, the tolerance zones must be described into which the geometry must fit

- Zones are specified using special symbols placed in feature control frames

- Notation must be clear and unambiguous to both design and manufacturing groups, and allow for easy inspection for SPC (Statistical Process Control)

- Geometrical tolerances are applied over and above normal dimensional tolerances.

- Used when it is necessary to control more precisely the form or shape of some feature

- Usually added because of the particular duty\function that the part\feature has to perform.

- Geometrical tolerances are used to convey in a brief and precise manner complete geometrical requirements on engineering drawings.

- They should always be considered for surfaces which come into contact with other parts, especially when close tolerances are applied to the features concerned.

- No language barrier exists, as the symbols used are in agreement with published recommendations of the International Organization for Standardization (ISO) and have been internationally agreed.

## General rules

- The symbols used in GD&T (with additions) are shown in Fig. 15.1 and symbols relating to geometrical characteristics are shown in Fig. 15.2.

|  | Note: If two or more groups of features are shown on the same axis, they shall be considered to be a single pattern when not related to a datum. |
| --- | --- |
| Fig 15.1 GD&T symbols with additional modifiers | Fig 15.2 Geometric symbols |

- Terms used to describe the geometrical characteristics of one type of geometrical tolerance can control another form of geometrical error.

- For example, a positional tolerance can control

  - perpendicularity and straightness

  - parallelism

  - perpendicularity

  - angularity tolerances can control flatness.

- The use of geometrical tolerances does not involve or imply any particular method of manufacture or inspection.

- Geometrical tolerances, in keeping with international conventions, must be met regardless of feature size unless modified by one of the following conditions:

(a) Maximum material condition, denoted by the symbol M describes a part, which contains the maximum amount of material, i.e. the minimum size hole or the maximum size shaft.

(b) Least material condition, denoted by the symbol L describes a part, which contains the minimum amount of material, i.e. the maximum size hole or the minimum size shaft.

Definitions

- Limits The maximum and minimum dimensions for a given feature are known as the ‘limits’. For example, 20 0.1. The upper and lower limits of size are 20.1 mm and 19.9 mm respectively.

- Tolerance The algebraic difference between the upper and lower limit.

- Nominal dimension Limits and tolerances are based on ‘nominal dimensions’ which are target dimensions.

In practice there is no such thing as a nominal dimension, since no part can be manufactured to a theoretical

exact size. The limits referred to above can be set in two ways:

(a) unilateral limits – limits set wholly above or below the nominal size;

(b) bilateral limits – limits set partly above and partly below the nominal size.

- Geometrical tolerance These tolerances specify the maximum error of a component’s geometrical characteristic, over its whole dimensioned length or surface. Defining a zone in which the feature may lie does this.

- Tolerance zone A tolerance zone is the space in which any deviation of the feature must be contained.

- The tolerance applies to the whole extent of the considered feature unless otherwise specified.

Theoretically Exact Dimensions: Boxed Dimensions

| These dimensions are identified by enclosure in a rectangular box, e.g. 50 | EQUI-SPACED | 60 | Ø30 |
| --- | --- | --- | --- |

- These are commonly known as ‘Boxed Dimensions’.

- They define the true position of a hole, slot, boss profile, etc.

Boxed dimensions are never individually tolerance but are always accompanied by a positional or zone tolerance specified within the tolerance frame referring to the feature.

| Geometrical tolerances are indicated by stating the following details in compartments in a rectangular frame. (a) the characteristic symbol, for single or related features; (b) the tolerance value (i) preceded by Ø if the zone is circular or cylindrical, (ii) preceded by SØ if the zone is spherical; (c) Letter or letters identifying the datum or datum systems. |  |
| --- | --- |

## Examples of GD&T symbols, usage and testing techniques

| Concentricity |  |  |
| --- | --- | --- |
| A modern approch: laser scanning and a coordinate measurement machine |  |  |