INTERNATIONAL	ISO

STANDARD 	9001

Second edition

1994-07-01

____________________________________

Quality systems - Model for quality assurance in design, development, production, installation and servicing

Systėmes qualite - Modėle pour l'assurance de la qualitė en conception, dėveloppement production, installation et prestations associėes

Reference number

ISO 9001:1994(E)

ISO 9001:1994(E)

Contents

Page

1	Scope 	1

2	Normative reference 	………………1

3	Definitions 		1

4	Quality system requirements 		1

4.1	Management responsibility 		1

4.2	Quality system 	2

4.3	Contract review 	3

4.4	Design control 	3

4.5	Document and data control 	4

4.6	Purchasing 	5

4.7 Control of customer-supplied product 	5

4.8 Product identification and traceability 	6

4.9 Process control 	6

4.10 Inspection and testing 	6

4.11 Control of inspection, measuring and test equipment 	7

4.12 Inspection and test status 	8

4.13 Control of nonconforming product 	8

4.14 Corrective and preventive action 	8

4.15 Handling, storage, packaging, preservation and delivery 	9

4.16	Control of quality records 	9

4.17	Internal quality audits 	9

4.18	Training 	10

4.19	Servicing 	10

4.20	Statistical techniques 	10

ISO 1994

All  rights reserved. Unless otherwise specified, no part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including

photocopying and microfilm, without permission in writing from the publisher.

International Organization for Standardization

Case Postale 56 * CH-1211 Geneve 20 9 Switzerland

Printed in Switzerland

iso	ISO 9001:1994(E)

Annex

A	Bibliography	..................................................................... 11

ISO 9001:1994(E) 	ISO

Foreword

ISO (the International Organization for Standardization) is a worldwide federation of national standards bodies (ISO member bodies). The work of preparing International Standards is normally carried out through ISO technical committees. Each member body interested in a subject for which a technical committee has been established has the right to be represented on that committee. International organizations, governmental and non-governmental, in liaison with ISO, also take part in the work. ISO collaborates closely with the International Electrotechnical Commission (IEC) on all matters of electrotechnical standardization.

Draft International Standards adopted by the technical committees are circulated to the member bodies for voting. Publication as an International Standard requires approval by at least 75% of the member bodies casting a vote.

International Standard ISO 9001 was prepared by Technical Committee ISO/TC 176, Quality management and quality assurance, Subcommittee SC 2, Quality systems.

This second edition cancels and replaces the first edition (ISO 9001:1987), which has been technically revised.

Annex A of this International Standard is for information only.

Introduction

This International Standard is one of three International Standards dealing with quality system requirements that can be used for external quality assurance purposes. The quality assurance models, set out in the three International Standards listed below, represent three distinct forms of quality system requirements suitable for the purpose of a supplier demonstrating its capability, and for the assessment of the capability of a supplier by external parties.

a)	ISO 9001, Quality systems - Model for quality assurance in design, development, production, installation and servicing

- for use when conformance to specified requirements is to be assured by the supplier during design, development, production, installation and servicing.

b)	ISO 9002, Quality systems - Model for quality assurance in production, installation and servicing

- for use when conformance to specified requirements is to be assured by the supplier during production, installation and servicing.

c)	ISO 9003, Quality systems - Model for quality assurance in final inspection and test

- for use when conformance to specified requirements is to be assured by the supplier solely at final inspection and test.

It is emphasized that the quality system requirements specified in this International Standard, ISO 9002 and ISO 9003 are complementary (not alternative) to the technical (product) specified requirements. They specify requirements which determine what elements quality systems have to encompass, but it is not the purpose of these International Standards to enforce uniformity of quality systems. They are generic and independent of any specific industry or economic sector. The design and implementation of a quality system will be influenced by the varying needs of an organization, its particular objectives, the products and services supplied, and the processes and specific practices employed.

It is intended that these International Standards will be adopted in their present form, but on occasions they may need to be tailored by adding or deleting certain quality system requirements for specific contractual situations. ISO 9000-1 provides guidance on such tailoring as well as on selection of the appropriate quality assurance model, viz. ISO 9001, ISO 9002 or ISO 9003.

INTERNATIONAL STANDARD  	ISO 9001:1994(E)

Quality systems - Model for quality assurance in design, development, production, installation and servicing

1 Scope	ISO 8402:1994, Quality management and quality assurance - Vocabulary.

This International Standard specifies quality system

requirements for use where a supplier's capability to	3 Definitions

design and supply conforming product needs to be

demonstrated.	For the purposes of this International Standard, the

The requirements specified are aimed primarily at 	definitions given in ISO 8402 and the following defi-

achieving customer satisfaction by preventing non- 	nitions apply.

conformity at all stages from design through to ser-

vicing. 	3.1 product: Result of activities or processes.

This International Standard is applicable in situations 	NOTES

when

2 A product may include service, hardware, processed

materials, software or a combination thereof.

a) design is required and the product requirements

are stated principally in performance terms, or	3 A product can be tangible (e.g. assemblies or processed

they need to be established, and	materials) or intangible (e.g. knowledge or concepts), or a

combination thereof.

b) confidence in product conformance can be at- 	4 For the purposes of this International Standard, the term

tained by adequate demonstration of a supplier's	"product" applies to the intended product offering only and

capabilities in design, development, production,	not to unintended "by-products" affecting the environment.

installation and servicing. 	This differs from the definition given in ISO 8402.

NOTE 1 For informative references, see annex A.

3.2 tender: Offer made by a supplier in response to

an invitation to satisfy a contract award to provide

product.

2 Normative reference	3.3 contract: Agreed requirements between a sup-

plier and customer transmitted by any means.

The following standard contains provisions which,

through reference in this text, constitute provisions

of this International Standard. At the time of publi- 	4 Quality system requirements

cation, the edition indicated was valid. All standards

are subject to revision, and parties to agreements 	4.1 Management responsibility

based on this International Standard are encouraged

to investigate the possibility of applying the most re- 	4.1.1 Quality policy

cent edition of the standard indicated below. Mem-

bers of IEC and ISO maintain registers of currently 	The supplier's management with executive responsi-

valid International Standards. 	bility shall define and document its policy for quality,

ISO 9001:1994(E) 	ISO

including objectives for quality and its commitment to	NOTE 5 The responsibility of a management represen-

quality. The quality policy shall be relevant to the	tative may also include liaison with external parties on

supplier's organizational goals and the expectations	matters relating to the supplier's quality system.

and needs of its customers. The supplier shall ensure

that this policy is understood, implemented and	4.1.3 Management review

maintained at all levels of the organization.

The supplier's management with executive responsi-

bility shall review the quality system at defined inter-

4.1.2 Organization	vals sufficient to ensure its continuing suitability and

effectiveness in satisfying the requirements of this

International Standard and the supplier's stated quality

4.1.2.1 Responsibility and authority	policy and objectives (see 4.1.1). Records of such re

views shall be maintained (see 4.16).

The responsibility, authority and the interrelation of

personnel who manage, perform and verify work af-	4.2 Quality system

fecting quality shall be defined and documented, par-

ticularly for personnel who need the organizational	4.2.1 General

freedom and authority to:

a) initiate action to prevent the occurrence of any	The supplier shall establish, document and maintain

nonconformities relating to the product, process	a quality system as a means of ensuring that product

and quality system;	conforms to specified requirements. The supplier

shall prepare a quality manual covering the require-

b) identify and record any problems relating to the	ments of this International Standard. The quality

product, process and quality system;	manual shall include or make reference to the quality

system procedures and outline the structure of the

c) initiate, recommend or provide solutions through 	documentation used in the quality system.

designated channels;

NOTE 6 Guidance on quality manuals is given in

d)	verify the implementation of solutions; 	IS010013.

e) control further processing, delivery or installation	4.2.2 Quality system procedures

of nonconforming product until the deficiency or

unsatisfactory condition has been corrected.	The supplier shall

a) prepare documented procedures consistent with

4.1.2.2 Resources		the requirements of this International Standard

and the supplier's stated quality policy, and

The supplier shall identify resource requirements and

provide adequate resources, including the assignment	b) effectively implement the quality system and its

of trained personnel (see 4.18), for management,		documented procedures.

performance of work and verification activities includ-

ing internal quality audits.	For the purposes of this International Standard, the

range and detail of the procedures that form part of

the quality system shall be dependent upon the com-

4.1.2.3 Management representative	plexity of the work, the methods used, and the skills

and training needed by personnel involved in carrying

The supplier's management with executive responsi-	out the activity.

bility shall appoint a member of the supplier's own

management who, irrespective of other responsi-	NOTE 7 Documented procedures may make reference

bilities, shall have defined authority for 	to work instructions that define how an activity is

performed.

a)	ensuring that a quality system is established, im-	4.2.3 Quality planning

plemented and maintained in accordance with this

International Standard, and	The supplier shall define and document how the re-

quirements for quality will be met. Quality planning

b)	reporting on the performance of the quality Sys-	shall be consistent with all other requirements of a

tem to the supplier's management for review and	supplier's quality system and shall be documented in

as a basis for improvement of the quality system. 	a format to suit the supplier's method of operation.

ISO 	ISO 9001:1994(E)

The supplier shall give consideration to the following	verbal means, the supplier shall ensure that the

activities, as appropriate, in meeting the specified re-	order requirements are agreed before their ac-

quirements for products, projects or contracts: 	ceptance;

a)	the preparation of quality plans;	b) any differences between the contract or order re-

quirements and those in the tender are resolved;

b)	the identification and acquisition of any controls,

processes, equipment (including inspection and	c) the supplier has the capability to meet the con

test equipment), fixtures, resources and skills that		tract or order requirements.

may be needed to achieve the required quality;

4.3.3 Amendment to a contract

c)	ensuring the compatibility of the design, the pro

duction process, installation, servicing, inspection	The supplier shall identify how an amendment to a

and test procedures and the applicable documen-	contract is made and correctly transferred to the

tation;	functions concerned within the supplier's organiz

ation.

d)	the updating, as necessary, of quality control, in

spection and testing techniques, including the	4.3.4 Records

development of new instrumentation;

Records of contract reviews shall be maintained (see

e)	the identification of any measurement require-	4.16).

ment involving capability that exceeds the known

state of the art, in sufficient time for the needed

with capability to be developed;	NOTE 9 Channels for communication and interfaces

with the customer's organization in these contract matters

should be established

- the identification of suitable verification at appro-

priate stages in the realization of product; 	4.4  Design control

g)	the clarification of standards of acceptability for	4.4.1 General

all features and requirements, including those

which contain a subjective element;	The supplier shall establish and maintain documented

procedures to control and verify the design of the

h)	the identification and preparation of quality rec-	product in order to ensure that the specified require

ords (see 4.16). 	ments are met.

NOTE 8 The quality plans referred to [see 4.2.3a)] may	4.4.2 Design and development planning

be in the form of a reference to the appropriate documented

procedures that form an integral part of the supplier's qual-	The supplier shall prepare plans for each design and

ity system.	development activity. The plans shall describe or ref

erence these activities, and define responsibility for

4.3 Contract review	their implementation. The design and development

activities shall be assigned to qualified personnel

4.3.1 General	equipped with adequate resources. The plans shall be

updated as the design evolves.

The supplier shall establish and maintain documented

procedures for contract review and for the coordi-	4.4.3 Organizational and technical interfaces

nation of these activities.

Organizational and technical interfaces between dif-

4.3.2 Review	ferent groups which input into the design process

shall be defined and the necessary information docu-

Before submission of a tender, or the acceptance of	mented, transmitted and regularly reviewed.

a contract or order (statement of requirement), the

tender, contract or order shall be reviewed by the	4.4.4 Design input

supplier to ensure that:

Design input requirements relating to the product, in-

a) the requirements are adequately defined and	cluding applicable statutory and regulatory require-

documented; where no written statement of re-	ments, shall be identified, documented and their

quirement is available for an order received by	selection reviewed by the supplier for adequacy. In

ISO 9001:1994(E)	ISO

complete, ambiguous or conflicting requirements shall	4.4.8 Design validation

be resolved with those responsible for imposing

these requirements.	Design validation shall be performed to ensure that

product conforms to defined user needs and/or re-

Design input shall take into consideration the results	quirements.

of any contract review activities.

NOTES

4.4.5 Design output	11 Design validation follows successful design

verification (see 4.4.7).

Design output shall be documented and expressed in	12 Validation is normally performed under defined

terms that can be verified and validated against design	operating conditions.

input requirements.

13 Validation is normally performed on the final

product, but may be necessary in earlier stages prior to product

completion.

Design output shall:

a) meet the design input requirements;	14 Multiple validations may be performed if there are diff-

ferent intended uses.

b) contain or make reference to acceptance criteria;

c)	identify those characteristics of the design that	4.4.9 Design changes

are crucial to the safe and proper functioning of

the product (e.g. operating, storage, handling,	All design changes and modifications shall be identi-

maintenance and disposal requirements).	fied, documented, reviewed and approved by author-

ized personnel before their implementation.

Design output documents shall be reviewed before

release.

4.5 Document and data control

4.4.6 Design review

4.5.1 General

At appropriate stages of design, formal documented

reviews of the design results shall be planned and	The supplier shall establish and maintain documented

conducted. Participants at each design review shall	procedures to control all documents and data that re

include representatives of all functions concerned	late to the requirements of this International Standard

with the design stage being reviewed, as well as	including, to the extent applicable, documents of ex-

other specialist personnel, as required. Records of	ternal origin such as standards and customer

such reviews shall be maintained (see 4.16).	drawings.

4.4.7 Design verification	NOTE 15 Documents and data can be in the form of

Any type of media, such as hard copy or electronic media.

At appropriate stages of design, design verification	4.5.2 Document and data approval and Issue

shall be performed to ensure that the design stage

output meets the design stage input requirements.	The documents and data shall be reviewed and ap

The design verification measures shall be recorded	proved for adequacy by authorized personnel prior to

(see 4.16).	issue. A master list or equivalent document control

procedure  identifying the current revision status of

NOTE 10 In addition to conducting design reviews (see	documents shall be established and be readily avail-

design verification may include activities such as	able to preclude the use of invalid and/or obsolete

- performing alternative calculations,	documents.

- comparing the new design with a similar proven design,	This control shall ensure that:

if available,

- undertaking tests and demonstrations, and	a) the pertinent issues of appropriate documents are

available at all locations where operations essen-

tial to the effective functioning of the quality sys-

- reviewing the design stage documents before release.		tem are performed;

ISO 	ISO 9001:1994(E)

b)	invalid and/or obsolete documents are promptly	a)	the type, class, grade or other precise identifi-

removed from all points of issue or use, or other-		cation;

wise assured against unintended use;

b)	the title or other positive identification, and appli-

c)	any obsolete documents retained for legal and/or		cable issues of specifications, drawings, process

knowledge-preservation purposes are suitably			requirements, inspection instructions and other

identified.			relevant technical data, including requirements for

approval or qualification of product, procedures,

4.5.3 Document and date changes 			process equipment and personnel;

Changes to documents and data shall be reviewed	c) the title, number and issue of the quality system

standard to be applied.

and approved by the same functions/organizations

that performed the original review and approval, un-	The supplier shall review and approve purchasing

less specifically designated otherwise. The desig-	documents for adequacy of the specified require-

nated functions/organizations shall have access to	ments prior to release.

pertinent background information upon which to base

their review and approval.	4.6.4 Verification of purchased product

Where practicable, the nature of the change shall be	4. 6.4.1 Supplier verification at subcontractor’s

identified in the document or the appropriate attach-	promises

ments.

Where the supplier proposes to verify purchased

4.6 Purchasing	product at the subcontractor ' 's premises, the supplier

shall specify verification arrangements and the

4.6.1 General	method of product release in the purchasing docu-

ments.

The supplier shall establish and maintain documented

procedures to ensure that purchased product (see	4.6.4.2 Customer verification of subcontracted

3.1) conforms to-specified requirements.	product

4.6.2 Evaluation of subcontractors	Where specified in the contract, the supplier's cus-

tomer or the customer's representative shall be af-

The supplier shall:	forded the right to verify at the subcontractor's

premises and the supplier's premises that subcon-

tracted product conforms to specified requirements.

a)	evaluate and select subcontractors on the basis	such verification shall not be used by the supplier as

of their ability to meet subcontract requirements	evidence of effective control of quality by the sub

including the quality system and any spec-	contractor.

ific quality assurance requirements;

Verification by the customer shall not absolve the

b)	define the type and extent of control exercised by	supplier of the responsibility to provide acceptable

the supplier over subcontractors. This shall be	product, nor shall it preclude subsequent rejection by

dependent upon the type of product, the impact	the customer.

of subcontracted product on the quality of final

product and, where applicable, on the quality audit

reports and/or quality records of the previously	4.7 Control of customer-supplied product

demonstrated capability and performance of sub-

contractors;	The supplier shall establish and maintain documented

procedures for the control of verification, storage and

c) establish and maintain quality records of accept- 	maintenance of customer-supplied product provided

able subcontractors (see 4.16).	for incorporation into the supplies or for related ac-

tivities.  Any such product that is lost, damaged or is

4.6.3 Purchasing date 	otherwise unsuitable for use shall be recorded and

reported to the customer (see 4.16).

Purchasing documents shall contain data clearly de- 	Verification by the supplier does not absolve the cus-

scribing the product ordered, including where appli- 	tomer of the responsibility to provide acceptable

cable: 	product.

ISO 9001:1994(E) 	ISO

4.8 Product identification and traceability	NOTE 16 Such processes requiring pre-qualification of

their process capability are frequently referred to as

special processes.

Where appropriate, the supplier shall establish and

maintain documented procedures for identifying the

product by suitable means from receipt and during all	Records shall be maintained for qualified processes,

stages of production, delivery and installation.	equipment and personnel, as appropriate (see 4.16).

Where and to the extent that traceability is a specified

requirement, the supplier shall establish and maintain	4.10 Inspection and testing

documented procedures for unique identification of

individual product or batches. This identification shall	4.10.1 General

be recorded (see 4.16).

The supplier shall establish and maintain documented

4.9 Process control	procedures for inspection and testing activities in or-

der to verify that the specified requirements for the

The supplier shall identify and plan the production, in-	product are met. The required inspection and testing,

stallation and servicing processes which directly af-	and the records to be established, shall be detailed in

fect quality and shall ensure that these processes are	the quality plan or documented procedures.

carried out under controlled conditions. Controlled

conditions shall include the following:	4.10.2 Receiving Inspection and testing

a)	documented procedures defining the manner of

production, installation and servicing, where the	4.10.2.1 The supplier shall ensure that incoming

absence of such procedures could adversely af-	product is not used or processed (except in the cir-

fect quality;	cumstances described in 4.10.2.3) until it has been

inspected or otherwise verified as conforming to

b)	use of suitable production, installation and servic-	specified requirements. Verification of conformance

ing equipment, and a suitable working environ-	to the specified requirements shall be in accordance

ment; 	with the quality plan and/or documented procedures.

c) 	compliance with reference standards/codes, qual-	4.10.2.2 In determining the amount and nature of

ity plans and/or documented procedures; 	receiving inspection, consideration shall be given to

the amount of control exercised at the subcontractor's

d)	monitoring and control of suitable process par- 	premises and the recorded evidence of conformance

ameters and product characteristics;	provided.

e) the approval of processes and equipment, as ap-	4.10.2.3 Where incoming product is released for ur-

propriate;	gent production purposes prior to verification, it shall

be positively identified and recorded (see 4.16) in or-

f)	criteria for workmanship, which shall be stipulated 	der to permit immediate recall and replacement in the

in the clearest practical manner (e.g. written 	event of nonconformity to specified requirements.

standards, representative samples or illustrations);

g) suitable maintenance of equipment to ensure	4.10.3 In-process inspection and testing

continuing process capability.

The supplier shall:

Where the results of processes cannot be fully veri-

fied by subsequent inspection and testing of the	a)	inspect and test the product as required by the

product and where, for example, processing de-	quality plan and/or documented procedures;

ficiencies may become apparent only after the prod-

uct is in use, the processes shall be carried out by

qualified operators and/or shall require continuous	b)	hold product until the required inspection and

monitoring and control of process parameters to en-		tests have been completed or necessary reports

sure that the specified requirements are met.		have been received and verified, except when

product is released under positive-recall pro-

The requirements for any qualification of process op-		cedures (see 4.10.2.3). Release under positive

erations, including associated equipment and person-		recall procedures shall not preclude the activities

nel (see 4.18), shall be specified. 		outlined in 4.10.3 a).

ISO 	ISO 9001:1994(E)

4.10.4 Final inspection and testing	quency of such checks and shall maintain records as

evidence of control (see 4.16).

The supplier shall carry out all final inspection and

testing in accordance with the quality plan and/or 	Where the availability of technical data pertaining to

documented procedures to complete the evidence of	the inspection, measuring and test equipment is a

conformance of the finished product to the specified	specified requirement, such data shall be made avail

requirements.	able, when required by the customer or customer's

representative, for verification that the inspection,

The quality plan and/or documented procedures for	measuring and test equipment is functionally ade-

final inspection and testing shall require that all	quate.

specified inspection and tests, including those speci

fied either on receipt of product or in-process, have	NOTE 17 For the purposes of this International

been carried out and that the results meet specified	Standard, the term "measuring equipment" includes

measurement requirements. 	devices.

No product shall be dispatched until all the activities	4.11.2 Control procedure

specified in the quality plan and/or documented pro-

cedures have been satisfactorily completed and the	The supplier shall:

associated data and documentation are available and

authorized.

a)	determine the measurements to be made and the

accuracy required, and select the appropriate in-

4.10.5 Inspection and test records		spection, measuring and test equipment that is

capable of the necessary accuracy and precision;

The supplier shall establish and maintain records

which provide evidence that the product has been in-	b)	identify all inspection, measuring and test equip-

spected and/or tested. These records shall show		ment that can affect product quality, and calibrate

clearly whether the product has passed or failed the		and adjust them at prescribed intervals, or prior to

inspections and/or tests according to defined accept-		use, against certified equipment having a known

ance criteria. Where the product fails to pass any in-		valid relationship to internationally or nationally

spection and/or test, the procedures for control of		recognized standards. Where no such standards

nonconforming product shall apply (see 4.13). 		exist, the basis used for calibration shall be docu-

mented;

Records shall identify the inspection authority re-

sponsible for the release of product (see 4.16).	c) 	define the process employed for the calibration

of inspection, measuring and test equipment, in-

4.11 Control of inspection, measuring and 	cluding details of equipment type, unique identifi-

test equipment		cation, location, frequency of checks, check

method, acceptance criteria and the action to be

taken when results are unsatisfactory;

4.11.1 General

d) 	identify inspection, measuring and test equipment

The supplier shall establish and maintain documented		with a suitable indicator or approved identification

procedures to control, calibrate and maintain in-		record to show the calibration status;

spection, measuring and test  equipment (including

test software) used by the supplier to demonstrate	e)	maintain calibration records for inspection,

the conformance of product to the specified require-		measuring and test equipment (see 4.16);

ments. Inspection, measuring and test equipment

shall be used in a manner which ensures that the	f)	assess and document the validity of previous in-

measurement uncertainty is known and is consistent		spection and test results when inspection,

with the required measurement capability. 		measuring or test equipment is found to be out

of calibration;

Where test software or comparative references such

as test hardware are used as suitable forms of in-	g)	ensure that the environmental conditions are

spection, they shall be checked to prove that they are		suitable for the calibrations, inspections,

capable of verifying the acceptability of product, prior		measurements and tests being carried out;

to release for use during production, installation or

servicing, and shall be rechecked at prescribed inter-	h)	ensure that the handling, preservation and storage

vals. The supplier shall establish the extent and fre-		of inspection, measuring and test equipment is

ISO 9001:1994(E)

ISO

such that the accuracy and fitness for use are	Where required by the contract, the proposed use or

maintained;	repair of product [see 4.13.2 b)] which does not con

form to specified requirements shall be reported for

i)	safeguard inspection, measuring and test facili-	concession to the customer or customer's represen-

ties, including both test hardware and test soft-	tative. The description of the nonconformity that has

ware, from adjustments which would invalidate	been accepted, and of repairs, shall be recorded to

the calibration setting.	denote the actual condition (see 4.16).

NOTE 18 The metrological confirmation system for	Repaired and/or reworked product shall be re

measuring equipment given in ISO 10012 may be used	inspected in accordance with the quality plan and/or

for guidance. 	documented procedures.

4.12 Inspection and test status	4.14 Corrective and preventive action

The inspection and test status of product shall be	4.14.1 General

identified by suitable means, which indicate the con-

formance or nonconformance of product with regard	The supplier shall establish and maintain documented

to inspection and tests performed. The identification	procedures for implementing corrective and preven--

of inspection and test status shall be maintained, as	tive action.

defined in the quality plan and/or documented pro-

cedures, throughout production, installation and ser-	Any corrective or preventive action taken to eliminate

vicing of the product to ensure that only product that	the causes of actual or potential nonconformities shall

has passed the required inspections and tests [or re-	be to a degree appropriate to the magnitude of prob-

leased under an authorized concession (see 4.13.2)	lems and commensurate with the risks encountered.

is dispatched, used or installed.

The supplier shall implement and record any changes

to the documented procedures resulting from correc-

4.13 Control of nonconforming product	tive and preventive action.

4.13.1 General	4.14.2 Corrective action

The supplier shall establish and maintain documented	The procedures for corrective action shall include:

procedures to ensure that product that does not con

form to specified requirements is prevented from un-	a) the effective handling of customer complaints and

intended use or installation. This control shall provide		reports of product nonconformities;

for identification, documentation, evaluation, segre-

gation (when practical), disposition of nonconforming	b) investigation of the cause of nonconformities re-

product, and for notification to the functions con-		lating to product, process and quality system, and

cerned.		recording the results of the investigation (see

4.16);

4.13.2 Review and disposition of nonconforming	c) determination of the corrective action needed to

product 		eliminate the cause of nonconformities;

The responsibility for review and authority for the	d) application of controls to ensure that corrective

disposition of nonconforming product shall be de-		action is taken and that it is effective.

Fined.

Nonconforming product shall be reviewed in accord-	4.14.3 Preventive action

ance with documented procedures. It may be

The procedures for preventive action shall include:

- reworked to meet the specified requirements,

a) the use of appropriate sources of information

b) accepted with or without repair by concession,		such as processes and work operations which af-

fect product quality, concessions, audit results,

c) regraded for alternative applications, or		quality records, service reports and customer

complaints to detect, analyse and eliminate po-

d) rejected or scrapped.		tential causes of nonconformities;

ISO 	ISO 9001:1994(E)

b)	determination of the steps needed to deal with	4.16 Control of quality records

any problems requiring preventive action;

The supplier shall establish and maintain documented

c)	initiation of preventive action and application of	procedures for identification, collection, indexing, ac-

controls to ensure that it is effective; 	cess, filing, storage, maintenance and disposition of

quality records.

d) ensuring that relevant information on actions

taken is submitted for management review (see 	Quality records shall be maintained to demonstrate

4.1.3).	conformance to specified requirements and the ef-

fective operation of the quality system. Pertinent

4.15 Handling, storage, packaging, 	quality records from the subcontractor shall be an el-

preservation and delivery	ement of these data.

All quality records shall be legible and shall be stored

4.15.1 General 	and retained in such a way that they are readily re-

trievable in facilities that provide a suitable environ-

The supplier shall establish and maintain documented 	ment to prevent damage or deterioration and to

procedures for handling, storage, packaging, preser- 	prevent loss. Retention times of quality records shall

vation and delivery of product. 	be established and recorded. Where agreed

contractually, quality records shall be made available

4.15.2 Handling 	for evaluation by the customer or the customer's

representative for an agreed period.

The supplier shall provide methods of handling prod- 	NOTE 19 Records may be in the form of any 'type of

uct that prevent damage or deterioration. 	media, such as hard copy or electronic media.

4.15.3 Storage

The supplier shall use designated storage areas or	4.17 Internal quality audits

stock rooms to. prevent damage or deterioration of

product, pending use or delivery. Appropriate meth-	The supplier shall establish and maintain documented-,

ods for authorizing receipt to and dispatch from such	procedures for planning and implementing internal

areas shall be stipulated.	quality audits to verify whether quality activities and

related results comply with planned arrangements

In order to detect deterioration, the condition of	and to determine the effectiveness of the quality

product in stock shall be assessed at appropriate in-	system.

tervals.

Internal quality audits shall be scheduled on the basis

4.15.4 Packaging	of the status and importance of the activity to be au-

dited and shall be carried out by personnel indepen-

The supplier shall control packing, packaging and	dent of those having direct responsibility for the

marking processes (including materials used) to the	activity being audited.

extent necessary to ensure conformance to specified 	The results of the audits shall be recorded (see 4.16)

requirements.	and brought to the attention of the personnel having

responsibility in the area audited. The management

4.15.5 Preservation	personnel responsible for the area shall take timely

corrective action on deficiencies found during the au-

The supplier shall apply appropriate methods for	dit.

preservation and segregation of product when the

product is under the supplier's control.	Follow-up audit activities shall verify and record the

implementation and effectiveness of the corrective

4.15.6 Delivery	action taken (see 4.16).

The supplier shall arrange for the protection  of the	NOTES

quality of product after final inspection and test.

Where contractually specified, this protection shall be	20 The results of internal quality audits form an integral

extended to include delivery to destination.	part of the input to management review activities (see

4.1.3).

ISO 9001:1994(E) 	ISO

21 Guidance on quality system audits is given in	cedures for performing, verifying and reporting that

IS010011. 	the servicing meets the specified requirements.

4.18 Training	4.20 Statistical techniques

The supplier shall establish and maintain documented	4.20.1 identification  of need

procedures for identifying training needs and provide

for the training of all personnel performing activities	The supplier. shall identify the need for statistical

affecting quality. Personnel performing specific as-	techniques required for establishing, controlling and

signed tasks shall be qualified on the basis of appro-	verifying process capability and product character-

priate education, training and/or experience, as	istics.

required. Appropriate records of training shall be

maintained (see 4.16).

4.20.2 Procedures

4.19 Servicing	The supplier shall establish and maintain documented

procedures to implement and control the application

Where servicing is a specified requirement, the sup-	of the statistical techniques identified in 4.20.1.

plier shall establish and maintain documented pro

ISO	ISO 9001:1994(E)

Annex A

(informative)

Bibliography

[1] ISO 9000-1:1994, Quality management and	[6] ISO 10011-1: 1990, Guidelines for auditing qual-

quality assurance standards  Part 1: Guide-		ity systems  Part 1: Auditing.

lines for selection and use.

[7] ISO 10011-2:1991, Guidelines for auditing qual-

[2] ISO 9000-2:1993, Quality management and		ity systems  Part 2. Qualification criteria for

quality assurance standards - Part 2. Generic		quality systems auditors.

guidelines for the application of ISO 9001, ISO

9002 and ISO 9003.	[8] ISO 10011-3:1991, Guidelines for auditing qual-

ity systems - Part 3. Management of audit.

(3] ISO 9000-3:1991, Quality management and		programmes.

quality assurance standards - Part 3: Guide

lines for the application of ISO 9001 to the de-	[9] ISO 10012-1:1992, Quality assurance require-

velopment, supply and maintenance of		ments for measuring equipment - Part 1:

software.		Metrological confirmation system for measuring

equipment.

[4]	ISO 9002:1994, Quality systems – Model for	[10] ISO 10013:--1)Guidelines for developing quality

quality assurance in production, installation		manuals.

and servicing.

[5]	ISO 9003:1994, Quality systems - Model for	(11] ISO/TR 13425:-'), Guidelines for the selection

quality assurance in final inspection and test		of statistical methods in standardization and

specification.

________________

1) To be published.